| Title: | Window Lifter Controller |
| --- | --- |

1.- PURPOSE………………………………………………………………………….………………………..4

2.-DEFINITIONS AND ABREVIATIONS…………………………………………….……………………….4

3.-REALIZATION CONSTRAINTS AND TARGETS………………………………………………………..5

4.-SW CONCEPTUAL DESIGN……………………………………………………………………………….6

5.- SW Component internal breakdown……………………………………………………………………7

5.1.- Diagrams by component…………………………..…………………………………………..7

5.1.1.-Complet Diagram…………………………………………………………………………..….8

## 5.2.- Functional Decomposition…………………………………………………………………………….10

5.3.- Function Description and Dynamic Behavior……………………………………………………..12

## 5.3.1- Function voidappUpDown_void_manualUp()……………………………….……………………12

## 5.3.2.- Function void appUpDown_void_manualDown()…………………..……..……………………12

## 5.3.3.- Function T_U32 appUpDown_u32_PushUpButton()…………………...………………………13

## 5.3.4.-FunctionT_U32appUpDown_u32_PushDownButton()……….…………………………………14

## 5.3.5.- Function T_U32 appUpDown_u32_PushAntipinch()……………………………………………14

## 5.3.6.- Function void appUpDown_void_default_Leds(T_U32 led)………………..………….………15

## 5.3.7.- Function void appUpDown_void_clearGPIO()………………………………...…………….……16

## 5.3.8.- Function  void appUpDown_void_timer1()……………………………………..……………..…..16

5.3.9.- Function void appUpDown_void_timer0()………………………………………….…..…………17

## 5.3.10.- Function T_U32 appUpDown_u32_validation10ms()…………………………………..……….18

## 5.3.11.- Function T_U32 appUpDown_u32_validation500ms()…………………………………………18

## 5.3.12.- Function T_U32 halWinMov_u32_ValidateUpButton()………………………….………..……19

## 5.3.13.- Function T_U32 halWinMov_u32_ValidateUpButton()………………..………..………………20

## 5.3.14.- Function T_U32 halWinMov_u32_ValidateAntipinch()…………………………..……20

## 5.3.15.- Function void halWinMov_void_ToggleBlueLed(T_U32 u32_status)…………..……21

## 5.3.16.- Function void halWinMov_void_ToggleRedLed(T_U32 u32_status)…………………22

## 5.3.17.- Function void halWinMov_void_ToggleGreenLed(T_U32 u32_status)…………………22

## 5.3.18.- Function void halWinMov_void_ToggleLedBar#(T_U32 u32_status)……………...……23

## 5.3.19 .- Function void main_void_onetouchUp()……………..…………………………...……24

## 5.3.20.- Function void main_void_onetouchDown()…………………………………………...……25

## 5.3.21.- Function void main_void_behaviorUp()….…………………………………………………..25

## 5.3.22.- Function void main_void_behaviorDown()….…………………………………………………..25

## 5.3.23.- Function void main_void_Antipinch()….…………………………………………………..26

## 5.3.24.- Function void main_void_idleState()….…………………………………………………..26

| History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Issue status (Index) | Maturity/Date (draft/invalid/valid) (dd-mmm-yyyy) | Author Department | Check/Release Department | Description |
| 1.0 | Draft 04-0ct-09 | Miguel Garcia | Miguel Garcia | Creation of the document. |
| 2.0 | Draft 26-Oct-17 | Rodrigo Mortera | Rodrigo Mortera | Insert Diagrams. |
| 2.1 | Draft 27-Oct-17 | Rafael Sanchez | Rafael Sanchez | Add function descriptions. |
| 2.2 | Draft 28-Oct-17 | Rodrigo Mortera | Rodrigo Mortera | Updates Diagrams |
| 2.3 | Draft 29-Oct-17 | Rafael Sanchez | Rafael Sanchez | Updates function descriptions and Add flow chart to each function. |

Table of Contents

# Purpose

This document explains how the window lifter works and also describe each function through the layer programming method.

# Definitions and abbreviations

Definitions

| BlueLed | Define to identify the port for the Blue Led |
| --- | --- |
| RedLed | Define to identify the port for the Red Led |
| GreenLed | Define to identify the port for the Green Led |
| UpButton | Define to identify the port for the push button 12 |
| DownButton | Define to identify the port for the push button 13 |
| LedBar_1 | Define to identify the port for the Bar’s first Led |
| LedBar_2 | Define to identify the port for the Bar’s secondary Led |
| LedBar_3 | Define to identify the port for the Bar’s third Led |
| LedBar_4 | Define to identify the port for the Bar’s fourth Led |
| LedBar_5 | Define to identify the port for the Bar’s fifth Led |
| LedBar_6 | Define to identify the port for the Bar’s sixth Led |
| LedBar_7 | Define to identify the port for the Bar’s seventh Led |
| LedBar_8 | Define to identify the port for the Bar’s eighth Led |
| LedBar_9 | Define to identify the port for the Bar’s ninth Led |
| LedBar_10 | Define to identify the port for the Bar’s tenth Led |
| malGPIO_init_void_WDOG_disable | Disable watchdog |
| malGPIO_init_void_PORT_init() | Port init |
| SOSC_init_8MHz | Clock of 8Mhz |
| SPLL_init_160MHz | Clock of 160Mhz |
| NormalRUNmode_80MHz | Set the mode of the timer to normal with 8Mhz |
| malGPIO_init_void_LPIT0_init | Initialize timers |
| u32_lpit0_ch0_flag_counter | Variable to control timer 0 |
| u32_lpit0_ch1_flag_counter | Variable to control timer 1 |
| appUpDown_void_clearGPIO() | Function to turnoff/turnon BlueLed, GreenLed, RedLed and Ledbars |
| appUpDown_u32_PushDownButton() | Read if Down Button is pushed |
| appUpDown_u32_PushUpButton() | Read if Up Button is pushed |
| appUpDown_u32_validation10ms() | Validate if the button is pushed at least during 10 ms |
| appUpDown_u32_validation500ms() | Validate if the button is pushed at least during 500 ms |
| appUpDown_void_timer1() | Increase the variable u32_lpit0_ch1_flag_counter each 1 ms |
| appUpDown_void_default_Leds(T_U32 led) | Turn on/off Leds in a default phase |
| main_void_oneTouchUp() | Ascending turn on the LedBar leds with just one touch |
| main_void_oneTouchDown() | Descending turn on the LedBar leds with just one touch |
| main_void_behaviorUp() | Select the turn on leds behavior. |
| main_void_behaviorDown() | Select the turn off leds behavior. |
| main_void_antiPinch() | Do the antipinch action |
| main_void_idleState() | Put the program in an idle state |

Abbreviations

| CH1 | Channel1 |
| --- | --- |
| CH2 | Channel2 |
| R&D | Research and Develop |

References

| N° | Document name | Reference |
| --- | --- | --- |
| 1 | USER_GUIDE_S32K144 | Link |
| 2 | S32K14x Series Reference Manual |
| 3 | S32K1xx Data Sheet |
| 4 | AN5413, s32k14x Series Cookbook |

# Realization constraints and targets

The objective of the task is to achieve to lift and down a window car simulated by 10 red LedBar regarding to the customer request. The development of the project must be on the Development Kit Platform S32K144EVB NXP microcontroller provided by Continental R&D.

# SW Conceptual design

- SW Component internal breakdown

  - Diagrams by component

- Validate_Antipinch function to validate the push in Push_Antipinch function in App.

- Validate_UpButton function to validate the push in Push_UpButton function in App.

- Validate_DownButton function works to validate the push in Push_DownButton function in App.

- Toggle functions execute the functionality to turn on or turn off the led’s, called by either Manual_Up or Manual_Down functions in App.

- The Clear_Leds function Works to condition if the leds should to be on or off.

- The Clear_GPIO only Works to clear all the inputs and outputs used in the program.

## Complete Diagram

appUpDown_Timer0 enables the timer to count 400ms.

appUpDown_Timer1 enables the timer to count 1ms.

## Functional Decomposition

  - Function Description and Dynamic Behavior

## Function void appUpDown_void_manualUp()

| Description | Ascending Turn on the Led Bar using a switch statement, the Led number is proportional with s32_switchFlag value. |
| --- | --- |
| Return Value | void |
| Precondition | The s32_switchFlag has to be set with a value [-1 to 9] |
| Post condition | The LedBar is turned on. |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_manualDown()

| Description | Descending Turn on the Led Bar using a switch statement, the Led number is proportional with s32_switchFlag value. |
| --- | --- |
| Return Value | void |
| Precondition | The s32_switchFlag has to be set with a value [-1 to 9] |
| Post condition | The ledbar is turned off. |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_PushUpButton()

| Description | Calls halWinMov_u32_ValidateUpButton function, to know if the halWinMov_u32_ValidateUpButton()is true or false |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1 , 0 |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_PushDownButton()

| Description | Calls Calls halWinMov_u32_ValidateDownButton() function, to know if the Calls halWinMov_u32_ValidateDownButton()is true or false |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1 , 0 |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_PushAntipinch()

| Description | Calls halWinMov_u32_ValidateAntipinch() function, to know if the halWinMov_u32_ValidateUpButton()is true or false |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1 , 0 |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_default_Leds(T_U32 led)

| Description | Using T_U32 led, with a switch statement, select case 0,1,2,3 and default. case 0 → Turn off Blue, Green and Red Led. case 1 → Turn off Blue. case 2 → Turn off Green. case 3 → Turn on Red. default → Turn on Blue, Green and Red Led. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | input T_U32 |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_clearGPIO()

| Description | Turn off Blue Led, Green Led, Red Led and LedBar# |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function  void appUpDown_void_timer1()

| Description | After 40000 cycles has occurred increment u32_lpit0_ch1_flag_counter variable. (1 ms) |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The LPIT_MSR_TIF1_MASK timer should be configured and initialized |
| Post condition | The LPIT_MSR_TIF1_MASK timer has been cleared |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_timer0()

| Description | After 16 000 000 cycles has occurred increment u32_lpit0_ch0_flag_counter variable. (400 ms) |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The LPIT_MSR_TIF0_MASK timer should be configured and initialized |
| Post condition | The LPIT_MSR_TIF0_MASK timer has been cleared |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_validation10ms()

| Description | Check when u32_lpit0_ch1_flag_counter is equal or higher to 10. This mean that 10 ms are occurred. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The LPIT_MSR_TIF1_MASK timer should be configured and initialized, u32_lpit0_ch0_flag should be 0 at the beginning |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_validation500ms()

| Description | Check when u32_lpit0_ch1_flag_counter is equal or higher to 500. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The LPIT_MSR_TIF1_MASK timer should be configured and initialized, u32_lpit0_ch1_flag should be 0 at the beginning |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 halWinMov_u32_ValidateUpButton()

| Description | Check if PORT C12 value is in High or Low and returns 1 if is High and 0 if is Low |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The PORTD C12 should be configured as input |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 halWinMov_u32_ValidateUpButton()

| Description | Check if PORT C13 value is in High or Low and returns 1 if is High and 0 if is Low |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The PORTD C13 should be configured as input. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 halWinMov_u32_ValidateAntipinch()

| Description | Check if PORT E13 value is in High or Low and returns 1 if is High and 0 if is Low |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The PORTD E13 should be configured as input. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleBlueLed(T_U32 u32_status)

| Description | Write in PORT D0: - High if int status = 1. - Low if int status = 0. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The PORTD D0 should be configured as output. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleRedLed(T_U32 u32_status)

| Description | Write in PORT D15: - High if int status = 1. - Low if int status = 0. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The PORTD D15 should be configured as output. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleGreenLed(T_U32 u32_status)

| Description | Write in PORT D16: - High if int status = 1. - Low if int status = 0. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The PORTD D16 should be configured as output. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleLedBar#(T_U32 u32_status)

| Description | Write High or Low in Led number # |  |
| --- | --- | --- |
|  | LedBar_1 Write in PORT C7: - High if int status = 1. - Low if int status = 0. LedBar_2 Write in PORT B17: - High if int status = 1. - Low if int status = 0. LedBar_3 Write in PORT B14: - High if int status = 1. - Low if int status = 0. LedBar_4 Write in PORT B15: - High if int status = 1. - Low if int status = 0. LedBar_5 Write in PORT B16: - High if int status = 1. - Low if int status = 0. | LedBar_6 Write in PORT C14: - High if int status = 1. - Low if int status = 0. LedBar_7 Write in PORT C3: - High if int status = 1. - Low if int status = 0. LedBar_8 Write in PORT E16: - High if int status = 1. - Low if int status = 0. LedBar_9 Write in PORT E15: - High if int status = 1. - Low if int status = 0. LedBar_10 Write in PORT E14: - High if int status = 1. - Low if int status = 0. |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |  |
| Parameter 2..n |  |  |
| Return Value | void |  |
| Precondition | Should be configured as output: |  |
|  | PORT C7 PORT B17 PORT B14 PORT B15 PORT B16 | PORT C14 PORT C3 PORT E16 PORT E15 PORT E14 |
| Post condition |  |  |
| Error Conditions |  |  |

Dynamic Behavior

## Function void main_void_onetouchUp()

| Description | Calls appUpDown_void_ManualUp() inside a while to similuted the lift movement. And turn on all the led bar |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

## Function void main_void_onetouchDown()

| Description | Calls appUpDown_void_ManualUp() inside a while to similuted the lift movement. And turn on all the led bar |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

## Function void main_void_behaviorUp()

| Description | Select the behavior between Manual Up and OneTouchUp |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

## Function void main_void_behaviorDown()

| Description | Select the behavior between Manual Down and OneTouchDown |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

## Function void main_void_Antipinch()

| Description | If the flag antipinch is 1, the action antipinch is going to be activated. During 5 seconds it will not respond to any action. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

## Function void main_void_idleState()

| Description | Put the all the leds in Low. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |