| Title: | Window Lifter Controller |
| --- | --- |

1.- PURPOSE………………………………………………………………………….………………………..4

2.-DEFINITIONS AND ABREVIATIONS…………………………………………….……………………….4

3.-REALIZATION CONSTRAINTS AND TARGETS………………………………………………………..5

4.-SW CONCEPTUAL DESIGN……………………………………………………………………………….6

5.- SW Component internal breakdown……………………………………………………………………7

5.1.- Diagrams by component…………………………..…………………………………………..7

5.1.1.-Complet Diagram…………………………………………………………………………..….8

## 5.2.- Functional Decomposition…………………………………………………………………………….10

5.3.- Function Description and Dynamic Behavior……………………………………………………..12

## 5.3.1- Function voidappUpDown_void_manualUp()……………………………….……………………12

## 5.3.2.- Function void appUpDown_void_manualDown()…………………..……..……………………12

## 5.3.3.- Function T_U32 appUpDown_u32_PushUpButton()…………………...………………………13

## 5.3.4.-FunctionT_U32appUpDown_u32_PushDownButton()……….…………………………………14

## 5.3.5.- Function T_U32 appUpDown_u32_PushAntipinch()……………………………………………14

## 5.3.6.- Function void appUpDown_void_default_Leds(T_U32 led)………………..………….………15

## 5.3.7.- Function void appUpDown_void_clearGPIO()………………………………...…………….……16

## 5.3.8.- Function  void appUpDown_void_timer1()……………………………………..……………..…..16

5.3.9.- Function void appUpDown_void_timer0()………………………………………….…..…………17

## 5.3.10.- Function T_U32 appUpDown_u32_validation10ms()…………………………………..……….18

## 5.3.11.- Function T_U32 appUpDown_u32_validation500ms()…………………………………………18

## 5.3.12.- Function T_U32 halWinMov_u32_ValidateUpButton()………………………….………..……19

## 5.3.13.- Function T_U32 halWinMov_u32_ValidateUpButton()………………..………..………………20

## 5.3.14.- Function T_U32 halWinMov_u32_ValidateAntipinch()…………………………..……20

## 5.3.15.- Function void halWinMov_void_ToggleBlueLed(T_U32 u32_status)…………..……21

## 5.3.16.- Function void halWinMov_void_ToggleRedLed(T_U32 u32_status)…………………22

## 5.3.17.- Function void halWinMov_void_ToggleGreenLed(T_U32 u32_status)…………………22

## 5.3.18.- Function void halWinMov_void_ToggleLedBar#(T_U32 u32_status)……………...……23

## 5.3.19 .- Function void main_void_onetouchUp()……………..…………………………...……24

## 5.3.20.- Function void main_void_onetouchDown()…………………………………………...……25

## 5.3.21.- Function void main_void_behaviorUp()….…………………………………………………..26

## 5.3.22.- Function void main_void_behaviorDown()….…………………………………………………..26

## 5.3.23.- Function void main_void_Antipinch()….…………………………………………………..27

## 5.3.24.- Function void main_void_idleState()….…………………………………………………..27

| History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Issue status (Index) | Maturity/Date (draft/invalid/valid) (dd-mmm-yyyy) | Author Department | Check/Release Department | Description |
| 1.0 | Draft 04-0ct-09 | Miguel Garcia | Miguel Garcia | Creation of the document. |
| 2.0 | Draft 26-Oct-17 | Rodrigo Mortera | Rodrigo Mortera | Insert Diagrams. |
| 2.1 | Draft 27-Oct-17 | Rafael Sanchez | Rafael Sanchez | Add function descriptions. |
| 2.2 | Draft 28-Oct-17 | Rodrigo Mortera | Rodrigo Mortera | Updates Diagrams |
| 2.3 | Draft 29-Oct-17 | Rafael Sanchez | Rafael Sanchez | Updates function descriptions and Add flow chart to each function. |
| 3 | Draft 1-Nov.17 | Rodrigo Mortera | Rodrigo Mortera | Update Diagrams |
| 3-1 | Draft 2-Nov-17 | Rafael Sanchez | Rafael Sanchez | Update Diagrams |

Table of Contents

# Purpose

This document explains how the window lifter works and also describe each function through the layer programming method.

# Definitions and abbreviations

Definitions

| BlueLed | Define to identify the port for the Blue Led |
| --- | --- |
| RedLed | Define to identify the port for the Red Led |
| GreenLed | Define to identify the port for the Green Led |
| UpButton | Define to identify the port for the push button 12 |
| DownButton | Define to identify the port for the push button 13 |
| LedBar_1 | Define to identify the port for the Bar’s first Led |
| LedBar_2 | Define to identify the port for the Bar’s secondary Led |
| LedBar_3 | Define to identify the port for the Bar’s third Led |
| LedBar_4 | Define to identify the port for the Bar’s fourth Led |
| LedBar_5 | Define to identify the port for the Bar’s fifth Led |
| LedBar_6 | Define to identify the port for the Bar’s sixth Led |
| LedBar_7 | Define to identify the port for the Bar’s seventh Led |
| LedBar_8 | Define to identify the port for the Bar’s eighth Led |
| LedBar_9 | Define to identify the port for the Bar’s ninth Led |
| LedBar_10 | Define to identify the port for the Bar’s tenth Led |
| malGPIO_init_void_WDOG_disable | Disable watchdog |
| malGPIO_init_void_PORT_init() | Port init |
| SOSC_init_8MHz | Clock of 8Mhz |
| SPLL_init_160MHz | Clock of 160Mhz |
| NormalRUNmode_80MHz | Set the mode of the timer to normal with 8Mhz |
| malGPIO_init_void_LPIT0_init | Initialize timers |
| u32_lpit0_ch0_flag_counter | Variable to control timer 0 |
| u32_lpit0_ch1_flag_counter | Variable to control timer 1 |
| appUpDown_void_clearGPIO() | Function to turnoff/turnon BlueLed, GreenLed, RedLed and Ledbars |
| appUpDown_u32_PushDownButton() | Read if Down Button is pushed |
| appUpDown_u32_PushUpButton() | Read if Up Button is pushed |
| appUpDown_u32_validation10ms() | Validate if the button is pushed at least during 10 ms |
| appUpDown_u32_validation500ms() | Validate if the button is pushed at least during 500 ms |
| appUpDown_void_timer1() | Increase the variable u32_lpit0_ch1_flag_counter each 1 ms |
| appUpDown_void_default_Leds(T_U32 led) | Turn on/off Leds in a default phase |
| main_void_oneTouchUp() | Ascending turn on the LedBar leds with just one touch |
| main_void_oneTouchDown() | Descending turn on the LedBar leds with just one touch |
| main_void_behaviorUp() | Select the turn on leds behavior. |
| main_void_behaviorDown() | Select the turn off leds behavior. |
| main_void_antiPinch() | Do the antipinch action |
| main_void_idleState() | Put the program in an idle state |

Abbreviations

| CH1 | Channel1 |
| --- | --- |
| CH2 | Channel2 |
| R&D | Research and Develop |

References

| N° | Document name | Reference |
| --- | --- | --- |
| 1 | USER_GUIDE_S32K144 | Link |
| 2 | S32K14x Series Reference Manual |
| 3 | S32K1xx Data Sheet |
| 4 | AN5413, s32k14x Series Cookbook |

# Realization constraints and targets

The objective of the task is to achieve to lift and down a window car simulated by 10 red LedBar regarding to the customer request. The development of the project must be on the Development Kit Platform S32K144EVB NXP microcontroller provided by Continental R&D.

# SW Conceptual design

Window lifter block diagram

- SW Component internal breakdown

  - Diagrams by component

Blue Led.

Lines and Functions related with Down behavior and green led.

Lines and Functions related with Up behavior.

Lines and Functions related with Antipinch behavior.

Red Led.

Timer functions and void function.

Status output port led’s function.

- Validate_Antipinch function validates the press in Push_Antipinch function in App.

- Validate_UpButton function  validates the press in Push_UpButton function in App.

- Validate_DownButton function works to validate the press in Push_DownButton function in App.

## s

- Toggle functions execute the functionality to turn on or turn off the led’s, called by either Manual_Up or Manual_Down functions in App.

- The default_Leds function Works to condition if the leds should be turned on or turned off.

Complete Diagram

- The Clear_GPIO only Works to clear all the outputs to initialize as turned off the led bar and the RGB led.

5.1.1 Complete Diagram

appUpDown_Timer0 enables the timer to count 400ms.

appUpDown_Timer1 enables the timer to count 1ms.

## Functional Decomposition

Black Line makes reference to appUpDown_void_clearGPIO() function.

Blue Line makes reference to appUpDown_void_default_Leds() function.

Black Line makes reference to appUpDown_void_ManualUp() function.

Blue Line makesreference to appUpDown_void_ManualDown() function.

  - Function Description and Dynamic Behavior

## Function void appUpDown_void_manualUp()

| Description | Ascending Turn on the Led Bar using a switch statement, the Led number is proportional with s32_switchFlag value. |
| --- | --- |
| Return Value | void |
| Precondition | The s32_switchFlag has to be set with a value [-1 to 9] |
| Post condition | The LedBar is turned on. |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_manualDown()

| Description | Descending Turn on the Led Bar using a switch statement, the Led number is proportional with s32_switchFlag value. |
| --- | --- |
| Return Value | void |
| Precondition | The s32_switchFlag has to be set with a value [-1 to 9] |
| Post condition | The ledbar is turned off. |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_PushUpButton()

| Description | Calls halWinMov_u32_ValidateUpButton function, to know if the halWinMov_u32_ValidateUpButton()is true or false |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1 , 0 |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_PushDownButton()

| Description | Calls Calls halWinMov_u32_ValidateDownButton() function, to know if the Calls halWinMov_u32_ValidateDownButton()is true or false |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1 , 0 |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_PushAntipinch()

| Description | Calls halWinMov_u32_ValidateAntipinch() function, to know if the halWinMov_u32_ValidateUpButton()is true or false |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1 , 0 |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_default_Leds(T_U32 led)

| Description | Using T_U32 led, with a switch statement, select case 0,1,2,3 and default. case 0 → Turn off Blue, Green and Red Led. case 1 → Turn off Blue. case 2 → Turn off Green. case 3 → Turn on Red. default → Turn on Blue, Green and Red Led. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | input T_U32 |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_clearGPIO()

| Description | Turn off Blue Led, Green Led, Red Led and LedBar# |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function  void appUpDown_void_timer1()

| Description | After 40000 cycles has occurred increment u32_lpit0_ch1_flag_counter variable. (1 ms) |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The LPIT_MSR_TIF1_MASK timer should be configured and initialized |
| Post condition | The LPIT_MSR_TIF1_MASK timer has been cleared |
| Error Conditions |  |

Dynamic Behavior

## Function void appUpDown_void_timer0()

| Description | After 16 000 000 cycles has occurred increment u32_lpit0_ch0_flag_counter variable. (400 ms) |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The LPIT_MSR_TIF0_MASK timer should be configured and initialized |
| Post condition | The LPIT_MSR_TIF0_MASK timer has been cleared |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_validation10ms()

| Description | Check when u32_lpit0_ch1_flag_counter is equal or higher to 10. This mean that 10 ms are occurred. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The LPIT_MSR_TIF1_MASK timer should be configured and initialized, u32_lpit0_ch0_flag should be 0 at the beginning |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 appUpDown_u32_validation500ms()

| Description | Check when u32_lpit0_ch1_flag_counter is equal or higher to 500. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The LPIT_MSR_TIF1_MASK timer should be configured and initialized, u32_lpit0_ch1_flag should be 0 at the beginning |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 halWinMov_u32_ValidateUpButton()

| Description | Check if PORT C12 value is in High or Low and returns 1 if is High and 0 if is Low |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The PORTD C12 should be configured as input |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 halWinMov_u32_ValidateUpButton()

| Description | Check if PORT C13 value is in High or Low and returns 1 if is High and 0 if is Low |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The PORTD C13 should be configured as input. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function T_U32 halWinMov_u32_ValidateAntipinch()

| Description | Check if PORT E13 value is in High or Low and returns 1 if is High and 0 if is Low |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | T_U32 1,0 |
| Precondition | The PORTD E13 should be configured as input. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleBlueLed(T_U32 u32_status)

| Description | Write in PORT D0: - High if int status = 1. - Low if int status = 0. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The PORTD D0 should be configured as output. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleRedLed(T_U32 u32_status)

| Description | Write in PORT D15: - High if int status = 1. - Low if int status = 0. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The PORTD D15 should be configured as output. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleGreenLed(T_U32 u32_status)

| Description | Write in PORT D16: - High if int status = 1. - Low if int status = 0. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |
| Parameter 2..n |  |
| Return Value | void |
| Precondition | The PORTD D16 should be configured as output. |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void halWinMov_void_ToggleLedBar#(T_U32 u32_status)

| Description | Write High or Low in Led number # |  |
| --- | --- | --- |
|  | LedBar_1 Write in PORT C7: - High if int status = 1. - Low if int status = 0. LedBar_2 Write in PORT B17: - High if int status = 1. - Low if int status = 0. LedBar_3 Write in PORT B14: - High if int status = 1. - Low if int status = 0. LedBar_4 Write in PORT B15: - High if int status = 1. - Low if int status = 0. LedBar_5 Write in PORT B16: - High if int status = 1. - Low if int status = 0. | LedBar_6 Write in PORT C14: - High if int status = 1. - Low if int status = 0. LedBar_7 Write in PORT C3: - High if int status = 1. - Low if int status = 0. LedBar_8 Write in PORT E16: - High if int status = 1. - Low if int status = 0. LedBar_9 Write in PORT E15: - High if int status = 1. - Low if int status = 0. LedBar_10 Write in PORT E14: - High if int status = 1. - Low if int status = 0. |
| Parameter 1 <input¦ output¦ inout> | Input T_U32 u32_status |  |
| Parameter 2..n |  |  |
| Return Value | void |  |
| Precondition | Should be configured as output: |  |
|  | PORT C7 PORT B17 PORT B14 PORT B15 PORT B16 | PORT C14 PORT C3 PORT E16 PORT E15 PORT E14 |
| Post condition |  |  |
| Error Conditions |  |  |

Dynamic Behavior

## Function void main_void_onetouchUp()

| Description | Calls appUpDown_void_ManualUp() inside a while to similuted the lift movement. And turn on all the led bar |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void main_void_onetouchDown()

| Description | Calls appUpDown_void_ManualUp() inside a while to similuted the lift movement. And turn on all the led bar |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void main_void_behaviorUp()

| Description | Select the behavior between Manual Up and OneTouchUp |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void main_void_behaviorDown()

| Description | Select the behavior between Manual Down and OneTouchDown |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void main_void_Antipinch()

| Description | If the flag antipinch is 1, the action antipinch is going to be activated. During 5 seconds it will not respond to any action. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior

## Function void main_void_idleState()

| Description | Put the all the leds in Low. |
| --- | --- |
| Parameter 1 <input¦ output¦ inout> |  |
| Parameter 2..n |  |
| Return Value | void |
| Precondition |  |
| Post condition |  |
| Error Conditions |  |

Dynamic Behavior